X100 air compressor comes with a 1.5 ft. stainless steel braided leader hose with 1/4” NPT swivel male fitting hose end and rubber isolated mounting feet as well as insulated electric terminals.

Perfect Compressor for Air Horn.

Specifications:

duty cycle: 15% @ 100 psi

flow rate: 1.34 cfm @ 0 psi

max working pressure: 130 psi

intermittent pressure: 245 psi

volt: 12v

amp: 16a

ingress protection rating: ip54

cylinder: Φ30mm

Accessories:

mounting hardware

vibration isolator

remote mount air filter assembly

stainless steel braided leader hose

check valve

| Performance Data |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PSI | CFM | A | BAR | LPM | A |
| 0 | 1.34 | 8.9 | 0 | 38 | 8.9 |
| 10 | 1.13 | 10.9 | 1 | 30 | 11.5 |
| 20 | 0.99 | 12.5 | 2 | 26.9 | 13.6 |
| 30 | 0.95 | 13.6 | 3 | 26.2 | 14.6 |
| 40 | 0.94 | 14.5 | 4 | 24.5 | 15.4 |
| 50 | 0.9 | 15.2 | 5 | 21.8 | 15.9 |
| 60 | 0.81 | 15.6 | 6 | 21.2 | 16 |
| 70 | 0.78 | 15.8 | 7 | 19.8 | 16.2 |
| 80 | 0.76 | 16 | 8 | 17.6 | 16.2 |
| 90 | 0.74 | 16.2 | 9 | 16.4 | 15.9 |
| 100 | 0.71 | 16.2 | 10 | 14.3 | 15 |
| 110 | 0.65 | 16.2 |  |  |  |
| 120 | 0.6 | 16.1 |  |  |  |
| 130 | 0.58 | 15.9 |  |  |  |
| 140 | 0.55 | 15.2 |  |  |  |
| 150 | 0.46 | 14.8 | *Supply Voltage: 13.8 Volts |  |  |

| RECOMMENDED COMPONENTS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 0.5 Gallon Tank |  |  |  |  |  |
| 1.0 Gallon Tank |  |  |  |  |  |
| 12V Relay |  |  |  |  |  |
| Pressure Switches |  |  |  |  |  |

Features:

- IP 54 Rated

- Gearless, direct-driven electric motors

- Oil-less design requires no maintenance (besides filter inspection/replacement)

- High performance and highly reinforced PTFE piston ring

- Anodized piston cylinder for increased wear protection

- Precision, high speed, durable bearings for increased piston & motor life

- Sealed motor housing for moisture resistance

- Environmental seals where electrical connections enter motor housing

- Efficient, oversized heat sink head assembly for increased heat dissipation, duty cycle & performance

- High performance check valve to eliminate restart pressure on head and piston

- Direct or remote mount option air filter assembly

- Replacement air filter elements (filtration to 10 microns)

- Stainless steel braided leader hose (PTFE lined where applicable)

- Heavy duty mounting feet (omega and standard styles, as applicable)

- Reinforced vibration isolators for safe & quiet operation

- Mounting hardware (bolts, washers, locking washers, nuts)

- Thermal Overload Protection (T.O.P. equipped compressors only) to protect compressor from overheating during normal operational duty cycle

- Maximum operating temperature of 158° F

- Minimum operating temperature of -40° F (unless otherwise noted)

- Wire terminal ends to simplify installation (ring ground terminal, spade positive lead terminal)