X400 compressor is a 33% duty cycle compressor that works quickly and efficiently putting out 2.54 CFM flow at 0 PSI.

It includes a standard 1/4” NPT stainless steel braided leader hose with inline check valve and insulated wiring for simple installation.

Air Suspension Compressor Assembly, Hard Mount Air Suspension High Pressure Air Pump Oil-free

Feature：

1.The motor is equipped with a self-restoring temperature protector.

2.Heavy-duty, direct-drive powerful motor.

3.Oil-free, shock pad, double filter, remote installation.

4.High Strength Stainless Steel Gas Valve.

5.High Performance Wear Resistant Piston Ring.

6.Anodized aluminum cylinder.

7.Durable air pump and motor with low noise, provide quiet work and long life.

8.moisture and dust resistant.

9.Heat-resisting covered steel yarn rubber tube with one-way valve and rotary joint.

Specifications:

12-Volt.

Permanent Magnetic Motor.

Duty Cycle: 33% @ 100 PSI.

Max. Working Pressure: 150PSI.

Max. Amp Draw: 26Amps.

Max. Flow Rate: 72L/Min (2.54CFM).

Net Weight: 8.75 lbs.

Max. Intermittent Pressure: 250PSI.

Dimensions: 217*100*171.5mm.

Maximum Restart Pressure: 200 PSI (13.8 bar)

Maximum Working Cycle: 33% (15m on / 30m off @ 100 PSI)

Sustainable Working Time: 40 minutes @ 30 PSI

Minimum Ambient Temperature: -40°F (-40° C)

Maximum Ambient Temperature: 176°F (80° C)

Parts Included:

- Air suspension compressor

- Check Valve

- Vibration Isolators

- Mounting Hardware

- Stainless Steel Braided Leader Hose

- Air Line for Remote Mount Air Filter

- Remote Mount Air Filter Assembly

- (2) Replacement Air Filter Elements

Key Words：

air suspension compressor

quiet air compressor heavy duty air pump

oil free compressor air ride compressor

air horn compressor portable air compressor pump

air suspension compressor pump car suspension compressor

Performance Data:

| PSI | CFM | Amp(s) |
| --- | --- | --- |
| 0 | 2.54 | 16 |
| 10 | 2.26 | 18 |
| 20 | 2.12 | 21 |
| 30 | 1.94 | 22 |
| 40 | 1.84 | 23 |
| 50 | 1.70 | 24 |
| 60 | 1.62 | 25 |
| 70 | 1.55 | 25 |
| 80 | 1.48 | 26 |
| 90 | 1.41 | 26 |
| 100 | 1.27 | 26 |
| 110 | 1.20 | 26 |
| 120 | 1.09 | 26 |
| 130 | 0.85 | 25 |
| 140 | 0.75 | 25 |
| 150 | 0.71 | 25 |

*Voltage: 13.8V