X400U compressor is a 33% duty cycle compressor that works quickly and efficiently putting out 2.68 CFM flow at 0 PSI.

It includes a standard 1/4” NPT stainless steel braided leader hose with inline check valve and insulated wiring for simple installation.

Model No. X400-U

Included Parts:

- Check Valve

- Vibration Isolators

- Mounting Hardware

- Stainless Steel Braided Leader Hose

- Air Line for Remote Mount Air Filter

- Remote Mount Air Filter Assembly

- (2) Replacement Air Filter Elements

SPECIFICATIONS:

- 12-Volt

- Permanent Magnetic Motor

- Duty Cycle: 33% @ 100 PSI

- Max. Working Pressure: 200 PSI

Max. Amp Draw: 35 Amps

Dimensions: 217*100*171.5mm

- Net Weight: 8.75 lbs.

Max. Intermittent Pressure: 250PSI

Max. Flow Rate: 76L/Min (2.68CFM) X400-U (DC-12V). PSI; LPM; Amp(s). 0; 76; 19.5. 10; 68; 21.3. 20; 66; 25.6. 30; 64; 27.4. 40; 62; 28.1. 50; 60; 29.3. 60; 58; 31.4. 70; 56; 32.1. 80; 54; 32.8. 90; 52; 33.4. 100; 50; 33.9. 110; 48; 34.2. 120; 46; 34.6. 130; 44; 34.8. 140; 43; 35. 150; 42; 35. Max. Pressure: more than 250psi.

2.68CFM @0PSI    33% DUTY @ 100PSI