X444 air compressor is a 100% duty cycle compressor that works quickly and efficiently putting out 1.76 CFM flow at 0 PSI.

It includes a standard 1/4” NPT stainless steel braided leader hose with inline check valve and insulated wiring for simple installation.

Features:

duty cycle: 100% @ 100 psi

flow rate: 1.76 cfm @ 0 psi

max working pressure: 200 psi

intermittent pressure: 300 psi

volt: 12v

amp: 23a

ingress protection rating: ip67

cylinder: Φ40mm

Accessories:

mounting hardware (nuts, bolts, locking washers)

vibration isolators

remote mount air filter assembly

two replacement air filter elements

air line for remote mount air filter installation

stainless steel braided leader hose

check valve

Parameter:

| PSI | CFM | A | BAR | LPM | A |
| --- | --- | --- | --- | --- | --- |
| 0 | 1.76 | 10 | 0 | 50 | 10 |
| 10 | 1.66 | 12 | 1 | 46 | 13 |
| 20 | 1.59 | 14 | 2 | 43 | 14 |
| 30 | 1.52 | 15 | 3 | 40.5 | 17 |
| 40 | 1.45 | 16 | 4 | 38.5 | 18 |
| 50 | 1.41 | 17 | 5 | 35.5 | 19 |
| 60 | 1.34 | 18 | 6 | 33 | 19 |
| 70 | 1.27 | 18 | 7 | 31.5 | 19 |
| 80 | 1.2 | 19 | 8 | 29.5 | 20 |
| 90 | 1.16 | 19 | 9 | 27 | 20 |
| 100 | 1.13 | 19 | 10 | 25 | 19 |
| 110 | 1.06 | 20 | 11 | 23 | 19 |
| 120 | 1.02 | 20 | 12 | 20.5 | 19 |
| 130 | 0.95 | 20 | 13 | 18 | 18 |
| 140 | 0.92 | 19 | 14 | 15.5 | 18 |
| 150 | 0.85 | 19 |  |  |  |
| 160 | 0.81 | 19 |  |  |  |
| 170 | 0.74 | 19 |  |  |  |
| 180 | 0.7 | 19 |  |  |  |
| 190 | 0.63 | 18 |  |  |  |
| 200 | 0.56 | 18 |  |  |  |

*Voltage: 13.8 V