X480U air compressor is a truly rated, 200 PSI compressor.

It features horizontal cooling fins and a trapezoidal head design, a standard 3/8” stainless steel braided leader hose with 3/8” NPT inline check valve and insulated wiring.

| Hardmount Air Compressor |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Model No. X480U |  |  |  |  |  |
| Included Parts |  |  |  | Specifications |  |
|  |  |  |  | 12-Volt |  |
| - Mounting hardware (nuts, bolts, locking washers) |  |  |  | Permanent Magnetic Motor |  |
| - Vibration isolators |  |  |  | Duty Cycle: 100% @ 100 PSI |  |
| Air filter assembly |  |  |  | Max. Working Pressure: 200 PSI |  |
| - Two replacement air filter elements |  |  |  | Max. Amp Draw: 33 Amps |  |
|  |  |  |  | Max. Flow Rate: 72L/Min (2.54CFM) |  |
| - Stainless steel braided leader hose |  |  |  | Dimensions: 254*100*171.5mm |  |
| - Check valve |  |  |  | Net Weight: 9.75lbs |  |
| Max. Intermittent Pressure : 250PSI |  |  |  |  |  |
| X480U(DC-12V) |  |  |  |  |  |
| PSI | LPM | Amp(s) |  |  |  |
| 0 | 72 | 14.2 |  |  |  |
| 20 | 65 | 17.6 |  |  |  |
| 40 | 58 | 20.6 |  |  |  |
| 60 | 54 | 26.2 |  |  |  |
| 80 | 50 | 28.9 |  |  |  |
| 100 | 47 | 30.1 |  |  |  |
| 120 | 42 | 31.7 |  |  |  |
| 140 | 40 | 32.8 |  |  |  |
| 150 | 38 | 33 |  |  |  |
| 160 | 34 | 33.1 |  |  |  |
| 200 | 30 | 32.5 |  |  |  |

## Standard Compressor Features:

- IP 67 Rated

- Gearless, direct-driven electric motors

- Oil-less design requires no maintenance

- High performance and highly reinforced PTFE piston ring

- Anodized piston cylinder for increased wear protection

- Precision, high speed, durable bearings for increased piston & motor life

- Sealed motor housing for moisture resistance

- Efficient, oversized heat sink head assembly for increased heat dissipation, duty cycle & performance

- Stainless steel braided leader hose

- Reinforced vibration isolators for safe & quiet operation

- Mounting hardware (bolts, washers, locking washers, nuts)

- Thermal Overload Protection to protect compressor from overheating during normal operational duty cycle

- Wire terminal ends to simplify installation (ring ground terminal, spade positive lead terminal)